# Project Documentation

# Title

Smart Bus & Road Safety Monitoring System with LoRa-Based Communication

# Abstract

This project integrates ESP32 microcontroller, GSM/GPS modules, ADXL345 accelerometer, ultrasonic sensors, and LoRa SX1278 transceivers to build a comprehensive bus safety monitoring system. The solution detects multiple accident scenarios, monitors passenger count, and provides real-time alerts via LCD, buzzer, GSM calls/SMS, and LoRa communication. The LoRa module enables long-range wireless signaling between vehicles, ensuring idle buses can detect nearby trucks and trigger safety alerts.

# Objectives

- Monitor passenger overcrowding and disable engine when unsafe.

- Detect driver alcohol consumption, smoke, flame, and rain conditions.

- Provide accident detection using ADXL345 accelerometer.

- Implement collision guard using ultrasonic sensor + servo.

- Enable GSM/GPS alerts with live location via SMS/calls.

- Integrate LoRa communication for inter-vehicle safety signaling.

- Display real-time status on LCD for user feedback.

# Hardware Components

- ESP32 microcontroller (main controller)

- ATmega328 (Arduino Uno/Nano) for LoRa prototype

- LoRa SX1278 transceiver (433 MHz)

- ADXL345 accelerometer (impact detection)

- Ultrasonic sensor (HC-SR04) for collision guard

- IR sensors for passenger entry/exit detection

- Alcohol sensor (MQ-3)

- Flame sensor, Smoke sensor

- Rain sensor

- Relay modules (engine, CO₂ sprinkler, glass breaker)

- LCD (20x4 I²C) for real-time display

- GSM (SIM800) & GPS modules for communication and tracking

- Buzzer + Servo motor for alerts

Hardware Components with Pin & Voltage Details

| Component | Operating Voltage | Key Pins | Integration Notes |
| --- | --- | --- | --- |
| ESP32 Microcontroller | 3.3V (5V via USB/VIN) | GPIO pins, 3.3V, GND | Central controller; use level shifters for 5V sensors. |
| ATmega328 (Arduino) | 5V | Digital D0– D13, Analog A0–A5 | Used for LoRa prototype interfacing. |
| LoRa SX1278 (433 MHz) | 3.3V | MOSI, MISO, SCK, NSS (SPI) | Requires 3.3V regulator; connect via SPI. |
| ADXL345 Accelerometer | 3.3V | SDA, SCL (I²C) or SPI pins | Detects impacts; typically uses I²C with ESP32. |
| Ultrasonic (HC- SR04) | 5V | VCC, GND, TRIG, ECHO | Warning: ECHO pin needs a voltage divider for ESP32. |
| IR Sensors | 5V | VCC, GND, OUT | Used for passenger entry/exit detection. |
| Alcohol Sensor (MQ-3) | 5V | VCC, GND, AO, DO | AO connects to ESP32 ADC for concentration. |
| Flame Sensor | 3.3–5V | VCC, GND, DO | Detects fire presence. |
| Gas Sensor (MQ- 2) | 5V | VCC, GND, AO, DO | Detects smoke/LPG. Needs preheating for stability. |
| Rain Sensor | 5V | VCC, GND, AO, DO | AO connects to ADC for intensity classification. |
| Relay Modules | 5V control | VCC, GND, IN | Controls engine cutoff and safety systems. |
| LCD (20x4 I²C) | 5V (3.3V Logic) | SDA, SCL | Displays passenger count, alerts, and speed. |
| GSM (SIM800) Module | 3.7–4.2V | TX, RX, VCC, GND | Use Li-ion battery; ensure UART level shifting. |
| GPS (NEO-6M) | 3.3–5V | TX, RX, VCC, GND | Provides live location data via UART. |
| Buzzer | 3.3–5V | VCC, GND, IN | Provides audible alerts. |

| Component | Operating Voltage | Key Pins | Integration Notes |
| --- | --- | --- | --- |
| Servo Motor | 5V | VCC, GND, PWM signal | Activates physical collision guards. |

# Software Tools

- Arduino IDE (programming ESP32 & ATmega328)

- Proteus (simulation of circuits)

- C/Embedded C (firmware development)

# System Architecture

- Input Layer: Sensors (IR, alcohol, flame, smoke, rain, ultrasonic, accelerometer).

- Processing Layer: ESP32 microcontroller integrates sensor data, applies safety logic, and triggers events.

- Output Layer: LCD display, buzzer, relays, GSM/GPS alerts, LoRa communication.

## Communication Layer:

  - GSM/GPS → sends SMS/calls with live location.

  - LoRa → transmits safety signals between vehicles (e.g., idle bus detects nearby truck).

# Working Principle

The Smart Bus & Road Safety Monitoring System operates as a continuous monitoring and control framework, integrating multiple sensors, actuators, and communication modules. The ESP32 microcontroller acts as the central hub, processing sensor inputs and executing safety logic in real time.

## Passenger Monitoring

  - IR sensors at the bus door detect entry and exit events.

  - The system maintains a live passenger count, displayed on the LCD.

  - If overcrowding is detected (count > 5), the engine relay is disabled, a buzzer alert is triggered, and a GSM SMS is sent to the control center.

  - When passenger count returns to safe levels, the engine relay is re-enabled.

## Driver Safety (Alcohol Detection)

  - The MQ-3 alcohol sensor continuously monitors the driver’s breath.

  - If alcohol concentration exceeds the threshold, the system gradually reduces engine speed using PWM control, preventing sudden stops.

  - A GSM call and SMS are sent to authorities with GPS location, and the LCD displays “ALCOHOL ALERT.”

## Fire and Smoke Detection

  - Flame and smoke sensors monitor the cabin environment.

  - On detection, the buzzer sounds, the CO₂ sprinkler relay activates, and the glass breaker relay is triggered for emergency evacuation.

  - GSM alerts are sent, and the LCD shows “FIRE ALERT” or “SMOKE ALERT” with sensor values.

## Rain-Based Speed Control

  - The rain sensor measures intensity and classifies conditions into low, moderate, or heavy rain.

  - Depending on rain severity, the system automatically reduces bus speed to safe limits (70, 65, or 60 km/h thresholds).

  - Alerts are displayed on the LCD and sent via GSM to notify the driver and control center.

## Collision Guard (Ultrasonic + Servo)

  - The ultrasonic sensor measures distance to vehicles ahead.

  - If another vehicle is detected within unsafe range, the servo guard is activated, buzzer sounds, and LCD shows “GUARD ALERT.”

  - Different thresholds are applied for low-speed (2 m) and high-speed (7 m) scenarios.

## Accident Detection (ADXL345 Accelerometer)

  - The accelerometer measures total acceleration across axes.

  - If sudden impact exceeds the threshold (12 g), the system identifies an accident.

  - Engine relay is cut off, buzzer sounds continuously, and GSM sends accident alerts with GPS coordinates.

## Communication Layer

  - GSM/GPS: Sends SMS/calls with live location to authorities during alcohol, fire, smoke, rain, or accident events.

  - LoRa: Provides long-range communication between buses and trucks. For example, an idle bus detects a nearby truck via LoRa and triggers LED alerts, enhancing roadside safety.

## User Feedback (LCD + Buzzer)

  - The LCD continuously displays passenger count, speed, and active alerts.

  - The buzzer provides immediate audible warnings for critical events.

  - Together, they ensure both driver and passengers are aware of the bus’s safety status.

# Results

- Successfully integrated multi-sensor accident detection with ESP32.

- Real-time alerts sent via LCD, buzzer, GSM SMS/calls.

- LoRa communication enabled long-range inter-vehicle signaling.

- System demonstrated improved road safety and passenger monitoring.

# Applications

- Public transport fleets (buses, school vans).

- Smart city infrastructure for accident prevention.

- Parking and idle vehicle monitoring using LoRa.

# Conclusion

The project demonstrates a multi-scenario bus safety monitoring system combining IoT, embedded systems, and wireless communication. By integrating GSM/GPS with LoRa, the solution ensures both local safety monitoring and long-range inter-vehicle alerts, making it suitable for smart city and fleet management applications.

# Future Scope

- Cloud integration for centralized monitoring.

- AI-based predictive accident detection.

- Expansion to multiple vehicles with mesh LoRa networks.

- Integration with government transport safety dashboards.